# Project Explanations

Verification of Thevenin’s Theorem

## Objective

To verify Thevenin’s Theorem by measuring full load current

## Theory

## Thevenin’s Theorem states that any linear, bilateral network consisting of multiple sources and resistors can be replaced by an equivalent circuit containing a single voltage source (Vth) in series with a single resistor  This simplification allows for easy analysis of the circuit's behavior when the load resistance (Rth) is changed.

## Procedure (Steps)

## Step 1: Original Circuit Analysis

## Build the Circuit: Place your components (Battery, R1,R2,R3,R4) and connect them

## Measure Load Current: Place a DC Ammeter in series with the load resistor . Run the simulation and record the current value as IL for "Circuit - I".

## Note: I have taken V as 15V.

Step 2: Finding Vth (Thevenin Voltage)

- Remove the Load: Delete the load resistor R4 and the ammeter from the circuit.

- Measure Open-Circuit Voltage: Place a DC Voltmeter across the open terminals where R4 used to be.

- Simulate: Run the simulation. The reading on the voltmeter is your Vth.

Step 3: Finding Rth (Thevenin Resistance)

- Deactivate Sources: Replace the DC Battery  with a short circuit (a plain wire).

- Measure Resistance: Use an Ohmmeter component connected across the open load terminals.

- Simulate: Run the simulation. The reading is your Rth .

Step 4: Thevenin Equivalent Circuit

- Build the Simplified Circuit: Place a new DC Voltage source and set its value to your calculated Vth (7.5V).

- Add Rth: Place a resistor in series with the source and set it to your calculated Rth 495 Ohm

- Reconnect Load: Connect the original load resistor R4 1 Ohm and a DC Ammeter in series.

- Verify: Run the simulation. The ammeter should show the same IL (approx 5.02 mA) as the original circuit.