# Project Explanations

## Project 1:

### Objective:

The purpose of this project is to measure current between two resistors and voltage across each resistor using Proteus simulation.

### Components Used:

- AC Voltage Source (5V)

- Resistors:

  - R1 = 5.1kΩ

  - R2 = 3.3kΩ

- Ammeter

- Voltmeter

- Ground

### Steps to Create Circuit in Proteus:

- Open Proteus software and click on New Project.

- Enter project name and proceed with default settings.

- Click on Pick Devices (P) button.

- Search and select the following components:

  - Resistor

  - AC Voltage Source

  - Ammeter

  - Voltmeter

  - Ground

- Place all components on the workspace.

- Set resistor values:

  - R1 = 5.1kΩ

  - R2 = 3.3kΩ

- Set AC source voltage to 5V.

- Connect the circuit in series using wires.

- Place the ammeter in series between R1 and R2.

- Connect the voltmeter in parallel across each resistor to measure voltage.

- Connect the ground terminal properly.

- Run the simulation and observe readings.

### Calculations:

Total Resistance:
R_total = R1 + R2
R_total = 5.1kΩ + 3.3kΩ = 8.4kΩ

Current (Ohm’s Law):
I = V / R
I = 5V / 8.4kΩ = 0.595 mA (approx)

### Voltage across Resistors:

V1 = I × R1
V1 = 0.595mA × 5.1kΩ ≈ 3.03V

V2 = I × R2
V2 = 0.595mA × 3.3kΩ ≈ 1.96V

## Project 2:

### Objective:

The purpose of this project is to design a simple series circuit and calculate current and voltage across each resistor.

### Components Used:

- DC Battery (9V)

- Resistors:

  - R1 = 3kΩ

  - R2 = 10kΩ

  - R3 = 5kΩ

- Connecting wires

### Steps to Create Circuit in Proteus:

- Open Proteus and create a New Project.

- Enter project details and continue with default options.

- Click on Pick Devices (P).

- Search and select:

  - Resistor

  - DC Battery

- Place components on the workspace.

- Set resistor values:

  - R1 = 3kΩ

  - R2 = 10kΩ

  - R3 = 5kΩ

- Set battery voltage to 9V.

- Connect all components in series (end-to-end) using wires.

- Check proper polarity of battery.

- Run simulation to verify circuit.

### Calculations:

Total Resistance:
R_total = R1 + R2 + R3
R_total = 3kΩ + 10kΩ + 5kΩ = 18kΩ

Current (Ohm’s Law):
I = V / R
I = 9V / 18kΩ = 0.5 mA

### Voltage across each resistor:

V1 = I × R1 = 0.5mA × 3kΩ = 1.5V
V2 = I × R2 = 0.5mA × 10kΩ = 5V
V3 = I × R3 = 0.5mA × 5kΩ = 2.5V