# Project Explanations

# Experiment 1: Verification of KCL (Kirchhoff’s Current Law)

## Objective

To verify Kirchhoff’s Current Law (KCL) by measuring currents in a circuit and showing that:

I1=I2+I3

## Theory

Kirchhoff’s Current Law states that the total current entering a junction is equal to the total current leaving the junction.

In this circuit, current I1I_1I1​ enters the node and splits into two currents I2​ and I3.

Therefore:

I1=I2+I3

## Procedure (Steps)

- Open Proteus software and create a new schematic.

- Place a DC Power Supply (RPS) in the circuit.

- Add resistors:

  - 330 Ω (left side)

  - 220 Ω (right side)

  - 1 kΩ (vertical branch)

  - 1 kΩ (right vertical)

- Connect the circuit exactly as shown in the diagram (forming a junction).

- Place ammeters:

  - A1 to measure I1

  - A2 to measure I2

  - A3 to measure I3

- Connect all components properly with ground.

- Set the power supply to 5V, run the simulation and note values of:

  - I1, I2, I3

- Repeat the same steps for:

  - 15V

  - 25V

- Record all readings in the table.

## Calculations (Using Ohm’s Law)

First find equivalent resistances:

### Branch 1 (I2 path):

R2=1kΩ

### Branch 2 (I3 path):

R3=220+1k=1220Ω

### Total parallel resistance:

1/Req=1/1000+1/1220

Then total resistance:

RT=330+Req

### Current Calculation

I1=V/RT

Then using current division:

I2=I1×R3/(R2+R3)

I3​=I1​×R2​/(R3+​R2)​​

## Result

It is verified that:

I1≈I2+I3

Hence, KCL is verified.

# Experiment 2: Verification of KVL (Kirchhoff’s Voltage Law)

## Objective

To verify Kirchhoff’s Voltage Law (KVL) by measuring voltages and showing that:

E1=V1+V2+V3

## Theory

Kirchhoff’s Voltage Law states that the sum of all voltages in a closed loop is equal to the source voltage.

## Procedure (Steps)

- Open Proteus software and create a new circuit.

- Place a DC Power Supply (RPS).

- Add resistors:

  - 330 Ω

  - 220 Ω

  - 1 kΩ

- Connect all resistors in the circuit as shown (loop form).

- Place voltmeters:

  - V1 across 330 Ω

  - V2 across 1 kΩ

  - V3 across 220 Ω

- Connect ground properly.

- Set supply voltage to:

  - 5V, run simulation and note V1, V2, V3

- Repeat for:

  - 10V

  - 15V

- Record all readings.

## Calculations

Total resistance:

RT=330+1000+220=1550Ω

### Current in circuit:

I=V/RT

### Voltage drops:

V1=I×330

V2=I×1000

V3=I×220

## Verification

E1=V1+V2+V3

## Result

It is observed that the sum of voltage drops across all resistors is equal to the input voltage.

Hence, KVL is verified.